SIMATS ENGINEERING

# ASSESSMENT TOOL 1

## COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

## COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

# ANALYTICAL PROBLEM SOLVING

- A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

- A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

- An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

- A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

- A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formu la with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

Question 1

Given:

Network Address = 192.168.20.0/24

Subnet allocation:

- Lab A → /26

- Lab B → /27

- Lab C → /28

Solution:

Lab A (/26)

Subnet Mask = 255.255.255.192

Network Address = 192.168.20.0

Host Range = 192.168.20.1 – 192.168.20.62

Broadcast Address = 192.168.20.63

Usable Hosts

= 2^(32−26) − 2

= 2⁶ − 2

= 64 − 2

= 62 hosts

Lab B (/27)

Next available subnet starts at 192.168.20.64

Subnet Mask = 255.255.255.224

Network Address = 192.168.20.64

Host Range = 192.168.20.65 – 192.168.20.94

Broadcast Address = 192.168.20.95

Usable Hosts

= 2^(32−27) − 2

= 2⁵ − 2

= 30 hosts

Lab C (/28)

Next available subnet starts at 192.168.20.96

Subnet Mask = 255.255.255.240

Network Address = 192.168.20.96

Host Range = 192.168.20.97 – 192.168.20.110

Broadcast Address = 192.168.20.111

Usable Hosts

= 2^(32−28) − 2

= 2⁴ − 2

= 14 hosts

Final Answer

| Lab | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- |
| A | 192.168.20.0/26 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 62 |
| B | 192.168.20.64/27 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 30 |
| C | 192.168.20.96/28 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 14 |

Explanation

Subnetting divides a large network into smaller networks, making IP address allocation more efficient. Each lab gets its own subnet, reducing unnecessary network traffic and improving security. Every subnet has its own network address, broadcast address, and a fixed number of usable host addresses.

Question 2

Given

System 1

IP Address = 192.168.1.130

Subnet Mask = 255.255.255.192 (/26)

System 2

IP Address = 192.168.1.190

Subnet Mask = 255.255.255.192 (/26)

Step 1: Identify the subnet

A /26 subnet has a block size of 64.

The subnet ranges are:

- 0 – 63

- 64 – 127

- 128 – 191

- 192 – 255

Both IP addresses fall in the range 128 – 191.

Network Address

192.168.1.128

Broadcast Address

192.168.1.191

Valid Host Range

192.168.1.129 – 192.168.1.190

Usable Hosts

= 2^(32−26) − 2

= 64 − 2

= 62 hosts

Final Answer

System 1

- Network Address = 192.168.1.128

- Host Range = 192.168.1.129 – 192.168.1.190

- Broadcast Address = 192.168.1.191

System 2

- Network Address = 192.168.1.128

- Host Range = 192.168.1.129 – 192.168.1.190

- Broadcast Address = 192.168.1.191

Both systems belong to the same subnet, and both IP addresses are valid.

Explanation

Using the subnet mask, we can identify the subnet to which each IP belongs. Since both systems are in the same subnet, they can communicate directly without requiring a router. A /26 subnet provides 62 usable host addresses.

Question 3

Given

Network Address = 172.16.0.0/16

Department allocation:

- HR → /25

- Sales → /26

- IT → /27

- Admin → /28

HR Department (/25)

Network Address = 172.16.0.0

Host Range = 172.16.0.1 – 172.16.0.126

Broadcast Address = 172.16.0.127

Usable Hosts = 126

Sales Department (/26)

Network Address = 172.16.0.128

Host Range = 172.16.0.129 – 172.16.0.190

Broadcast Address = 172.16.0.191

Usable Hosts = 62

IT Department (/27)

Network Address = 172.16.0.192

Host Range = 172.16.0.193 – 172.16.0.222

Broadcast Address = 172.16.0.223

Usable Hosts = 30

Admin Department (/28)

Network Address = 172.16.0.224

Host Range = 172.16.0.225 – 172.16.0.238

Broadcast Address = 172.16.0.239

Usable Hosts = 14

Remaining Unused IP Range

The addresses from

172.16.0.240 onwards

remain unused and are available for future subnet allocation within the 172.16.0.0/16 network.

Final Answer

| Department | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- |
| HR | 172.16.0.0/25 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 126 |
| Sales | 172.16.0.128/26 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 62 |
| IT | 172.16.0.192/27 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 30 |
| Admin | 172.16.0.224/28 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 14 |

Explanation

Each department is assigned a subnet based on the required subnet mask. This ensures efficient utilization of IP addresses while keeping departments logically separated. The remaining address space can be used later if the organization expands its network.

Question 4

Given:

- Network Address = 172.16.10.0/24

- Subnet Mask = /27 (255.255.255.224)

- IP Address = 172.16.10.78

- Check whether 172.16.10.95 belongs to the same subnet.

Solution:

A /27 subnet has a block size of 32.

The subnets are:

- 172.16.10.0 – 172.16.10.31

- 172.16.10.32 – 172.16.10.63

- 172.16.10.64 – 172.16.10.95

- 172.16.10.96 – 172.16.10.127

- 172.16.10.128 – 172.16.10.159

- 172.16.10.160 – 172.16.10.191

- 172.16.10.192 – 172.16.10.223

- 172.16.10.224 – 172.16.10.255

The IP address 172.16.10.78 falls in the subnet 172.16.10.64 – 172.16.10.95.

Therefore,

Network Address = 172.16.10.64

Broadcast Address = 172.16.10.95

Valid Host Range = 172.16.10.65 – 172.16.10.94

Number of Subnets

Original network = /24

New subnet mask = /27

Borrowed bits = 3

Number of subnets

= 2³

= 8 subnets

Number of Usable Hosts

= 2^(32−27) − 2

= 2⁵ − 2

= 32 − 2

= 30 hosts

Checking IP Address 172.16.10.95

172.16.10.95 is the broadcast address of this subnet.

Therefore, it cannot be assigned to any host, but it does belong to the same subnet.

Final Answer

- Total subnets = 8

- Usable hosts per subnet = 30

- Network address = 172.16.10.64

- Broadcast address = 172.16.10.95

- Valid host range = 172.16.10.65 – 172.16.10.94

- IP 172.16.10.95 is the broadcast address, so it is not a valid host address.

Explanation

Subnetting divides a network into smaller sections, making IP address management easier and reducing unnecessary traffic. In this case, the given IP address belongs to the third subnet. Since 172.16.10.95 is the broadcast address, it is reserved for sending messages to all devices within that subnet and cannot be assigned to a computer.

Question 5

Given:

IPv6 Address

2001:0db8:0000:0000:0000:ff00:0042:8329

Prefix Length = /64

Solution:

Step 1: Compress the IPv6 Address

Remove leading zeros.

Replace consecutive groups of zeros with "::".

Compressed address:

2001:db8::ff00:42:8329

Step 2: Expand the Address

Expanded form:

2001:0db8:0000:0000:0000:ff00:0042:8329

Step 3: Network Prefix

For a /64 prefix, the first 64 bits represent the network.

Network Prefix:

2001:0db8:0000:0000::/64

Step 4: Number of Host Addresses

Host bits

= 128 − 64

= 64 bits

Number of host addresses

= 2⁶⁴

= 18,446,744,073,709,551,616

(approximately 18.4 quintillion addresses)

Final Answer

- Compressed IPv6 address = 2001:db8::ff00:42:8329

- Expanded IPv6 address = 2001:0db8:0000:0000:0000:ff00:0042:8329

- Network Prefix = 2001:0db8:0000:0000::/64

- Number of host addresses = 2⁶⁴ (18,446,744,073,709,551,616)

Explanation

IPv6 uses 128-bit addresses, which provide a much larger address space than IPv4. Compression removes unnecessary zeros, making the address shorter and easier to read. A /64 prefix is the standard size used in most IPv6 networks, leaving 64 bits for host addresses. This provides an extremely large number of unique addresses, making IPv6 suitable for modern networks and the growing number of Internet-connected devices.