SIMATS ENGINEERING

ASSESSMENT TOOL 2

# COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

# COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

    - Determine the network address and broadcast address for each department.

    - Identify the valid host IP address range for each subnet.

    - Calculate the number of usable host addresses available in each subnet.

    - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each department.

    - Identify the valid range of host IP addresses.

    - Calculate the number of usable hosts in each subnet.

    - Identify the unused IP address range remaining within the allocated IP block.

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

    - Convert the given IPv6 address into its compressed notation.

    - Expand the compressed address back into its full hexadecimal representation.

    - Identify the network prefix using the /64 prefix length.

    - Calculate the total number of host addresses supported within the subnet.

    - the total number of possible host addresses available within this network.

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

- A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

|  |  | Scenario Based Assessment |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning ) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned , logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |

Question 1

Given:

Network Address = 192.168.100.0/24

Subnet allocation:

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

Solution

School of Computing (/25)

Subnet Mask = 255.255.255.128

Network Address = 192.168.100.0

Host Range = 192.168.100.1 – 192.168.100.126

Broadcast Address = 192.168.100.127

Usable Hosts

= 2^(32−25) − 2

= 2⁷ − 2

= 126 hosts

School of Electronics (/26)

Next available subnet starts at 192.168.100.128

Network Address = 192.168.100.128

Host Range = 192.168.100.129 – 192.168.100.190

Broadcast Address = 192.168.100.191

Usable Hosts

= 2^(32−26) − 2

= 62 hosts

School of Mechanical Engineering (/27)

Next available subnet starts at 192.168.100.192

Network Address = 192.168.100.192

Host Range = 192.168.100.193 – 192.168.100.222

Broadcast Address = 192.168.100.223

Usable Hosts

= 2^(32−27) − 2

= 30 hosts

Remaining Unused IP Range

The addresses from

192.168.100.224 – 192.168.100.255

are still available for future expansion.

Explanation

Subnetting divides the network into smaller sections based on the number of devices required. This improves network performance, reduces unnecessary traffic, and allows each department to have its own independent subnet while efficiently utilizing IP addresses.

Question 2

Given

Network Address = 10.10.0.0/16

Subnet allocation:

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

Solution

Software Development (/24)

Network Address = 10.10.0.0

Host Range = 10.10.0.1 – 10.10.0.254

Broadcast Address = 10.10.0.255

Usable Hosts = 254

Quality Assurance (/25)

Network Address = 10.10.1.0

Host Range = 10.10.1.1 – 10.10.1.126

Broadcast Address = 10.10.1.127

Usable Hosts = 126

Human Resources (/26)

Network Address = 10.10.1.128

Host Range = 10.10.1.129 – 10.10.1.190

Broadcast Address = 10.10.1.191

Usable Hosts = 62

Executive Management (/27)

Network Address = 10.10.1.192

Host Range = 10.10.1.193 – 10.10.1.222

Broadcast Address = 10.10.1.223

Usable Hosts = 30

Remaining Unused IP Range

The remaining addresses from

10.10.1.224 onwards

are available for future departments within the 10.10.0.0/16 network.

Explanation

Different departments require different subnet sizes depending on the number of employees. Using different subnet masks helps allocate IP addresses efficiently while keeping each department isolated. The unused address range can be assigned later when the organization expands.

Question 3

Given

IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Solution

Compressed IPv6 Address

2001:db8:abcd::/64

Expanded IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000

Network Prefix

Since the prefix length is /64, the first 64 bits represent the network.

Network Prefix

2001:0db8:abcd:0000::/64

Number of Host Addresses

Host bits

= 128 − 64

= 64 bits

Number of host addresses

= 2⁶⁴

= 18,446,744,073,709,551,616

(approximately 18.4 quintillion addresses)

Explanation

IPv6 provides a much larger address space than IPv4. Compressing the address removes unnecessary zeros, making it easier to read, while expanding it restores the complete hexadecimal format. With a /64 prefix, the first half of the address identifies the network, and the remaining 64 bits are available for host devices. This large address space makes IPv6 ideal for modern networks with thousands of connected devices.

Question 4

Given:

Routing Table:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP Address = 192.168.2.45

Solution

The destination IP address is 192.168.2.45.

Since the first three octets are 192.168.2, the packet belongs to the subnet 192.168.2.0/24.

The matching routing table entry is:

192.168.2.0/24

The router forwards the packet through the interface or next-hop connected to the 192.168.2.0/24 network.

If the router does not find a matching entry in the routing table, it forwards the packet using the default route (0.0.0.0/0). If no default route is configured, the packet is discarded.

Final Answer

- Destination subnet = 192.168.2.0/24

- Matching routing table entry = 192.168.2.0/24

- Next-hop/interface = Interface connected to 192.168.2.0/24

- Default route = 0.0.0.0/0

Explanation

A router compares the destination IP address with every network in its routing table. When it finds a matching network, it forwards the packet through the corresponding interface. If there is no matching entry, the router uses the default route to forward the packet to another router. This process ensures that packets are delivered to the correct destination efficiently.

Question 5

Given:

Administration LAN = 192.168.10.0/26

Finance LAN = 192.168.10.64/26

Router Interfaces:

Administration Gateway = 192.168.10.1

Finance Gateway = 192.168.10.65

Administration LAN

Network Address = 192.168.10.0

Broadcast Address = 192.168.10.63

Host Range = 192.168.10.1 – 192.168.10.62

Suggested IP addresses for three computers:

- PC1 → 192.168.10.2

- PC2 → 192.168.10.3

- PC3 → 192.168.10.4

Default Gateway:

192.168.10.1

Finance LAN

Network Address = 192.168.10.64

Broadcast Address = 192.168.10.127

Host Range = 192.168.10.65 – 192.168.10.126

Suggested IP addresses for three computers:

- PC1 → 192.168.10.66

- PC2 → 192.168.10.67

- PC3 → 192.168.10.68

Default Gateway:

192.168.10.65

Final Answer

| LAN | Network Address | Host Range | Broadcast Address | Default Gateway |
| --- | --- | --- | --- | --- |
| Administration | 192.168.10.0/26 | 192.168.10.1 – 192.168.10.62 | 192.168.10.63 | 192.168.10.1 |
| Finance | 192.168.10.64/26 | 192.168.10.65 – 192.168.10.126 | 192.168.10.127 | 192.168.10.65 |

Suggested Host IP Addresses

Administration LAN

- PC1 – 192.168.10.2

- PC2 – 192.168.10.3

- PC3 – 192.168.10.4

Finance LAN

- PC1 – 192.168.10.66

- PC2 – 192.168.10.67

- PC3 – 192.168.10.68

Explanation

The router connects the Administration and Finance LANs using separate interfaces. Each LAN has its own network address, broadcast address, and valid host range. The default gateway for each LAN is the router interface connected to that network. Whenever a computer needs to communicate with another subnet, it sends the data to its default gateway, which forwards the packet to the correct destination. This setup allows both departments to communicate securely while remaining on separate networks.